# ECA1803 OPTOELECTRONICS AND OPTICAL COMMUNICATION

CO1 CIA-1 Report: Optical Fibre Link Evaluation

Name: Dinesh Kumar	Register No.: 192312169

## Q1. Evaluating Fiber Parameters for a Campus Monitoring Link [BL: K5] (10 marks)

A campus engineer is selecting a step-index fiber for a 1300 nm monitoring link driven by an LED source. The fiber has core index n₁ = 1.470, cladding index n₂ = 1.455 and core radius a = 25 μm. The supplier claims: "the fiber has a narrow acceptance cone, supports around 150 modes, and can be made single-mode at 1550 nm without changing the refractive indices." Evaluate each claim before selecting the fiber.

Summary. The fibre is multimode with NA ≈ 0.21 and about 320 modes, so the "150 modes" and "single-mode at 1550 nm" claims fail. It is still suitable for an LED link.

### Numerical aperture and acceptance angle

Evaluate the fiber's light-gathering capability by calculating its numerical aperture and acceptance angle in air, and judge whether it is suitable for LED coupling. (3)

NA = √(n₁² − n₂²) = √(1.470² − 1.455²) = √0.043875 = 0.2095

θmax = sin⁻¹(NA) = sin⁻¹(0.2095) = 12.1°

The full acceptance cone is about 24.2°. An LED is a wide-angle (Lambertian) emitter, so only part of its light is captured; coupling efficiency is roughly NA² ≈ 0.044 (about 4.4%, or −13.6 dB). This is typical for LED-to-multimode coupling and acceptable for a short monitoring link. Judgement: the cone is moderate, not unusually narrow, and the fibre is suitable for LED coupling.

### Normalised frequency and mode count at 1300 nm

Assess the supplier's mode-count claim using the normalized frequency at 1300 nm and an estimate of the guided-mode count. (3)

V = (2πa/λ)·NA = (2π × 25 μm / 1.3 μm) × 0.2095 = 25.3

M ≈ V²/2 = (25.3)² / 2 ≈ 320 modes

Judgement: the claim of about 150 modes is wrong. The fibre carries roughly 320 modes, more than twice the claimed number.

### Single-mode or multimode

Use the LP₁₁ cut-off condition to classify the fiber as single-mode or multimode, and judge its suitability for a 500 m campus monitoring link. (2)

The LP₁₁ cut-off requires V < 2.405 for single-mode operation. Here V = 25.3, far above 2.405, so the fibre is multimode. For a 500 m link, modal dispersion is estimated with Δ = (n₁ − n₂)/n₁

= 0.0102:

Δτ = L n₁ Δ / c = 500 × 0.015 / (3×10⁸) = 25 ns

This limits the bit rate to roughly 0.7/Δτ ≈ 28 Mb/s. Judgement: acceptable for a low-rate campus monitoring link driven by an LED, and the large core eases connectors and coupling.

### Single-mode at 1550 nm by reducing core radius

Evaluate the proposal to achieve single-mode operation at 1550 nm by reducing only the core radius. Estimate the maximum radius and recommend whether this is a practical design choice. (2)

a_max = 2.405 λ / (2π NA) = 2.405 × 1.55 / (2π × 0.2095) = 2.83 μm

At 1550 nm the present fibre has V ≈ 21.2, so the radius would have to shrink from 25 μm to under 2.83 μm. Recommendation: this is not a practical design choice. A core of about 5.7 μm diameter is very hard to couple with an LED, needs a laser source and tight connector and splice tolerances, and replaces the fibre with a different type altogether. A standard single-mode fibre with a smaller index difference is the better route if single-mode operation is ever needed.

## Q2. Link-Budget Assessment of a 25 km Access Link [BL: K5] (10 marks)

An engineer is assessing a 25 km single-mode access link. The fiber has absorption loss

0.06 dB/km, Rayleigh scattering loss 0.10 dB/km and bending loss 0.02 dB/km. The transmitter launches 2 mW. The link includes 3 connectors (0.4 dB each) and 4 fusion splices (0.08 dB each). Receiver sensitivity is −20 dBm and a 5 dB system margin is required. The team proposes approving the link and using the same terminal equipment for a 100 km route.

Summary. The 25 km link is acceptable: total loss is 6.02 dB, received power is −3.01 dBm (500 μW) and the margin over sensitivity is 17 dB. The same equipment cannot support 100 km, where the margin drops to 3.49 dB and the limit is about 91.6 km.

### Combined attenuation coefficient

Calculate the combined fiber attenuation coefficient and evaluate which loss mechanism contributes most and which may be reduced through careful installation. (2)

α = 0.06 + 0.10 + 0.02 = 0.18 dB/km

Rayleigh scattering (0.10 dB/km, about 56% of the total) contributes most, but it is intrinsic to the glass. Bending loss (0.02 dB/km) is the one that careful installation can reduce, through correct bend radius and cable handling. Absorption is a material property.

### Total loss over 25 km

Determine the total loss over 25 km, including fiber, connectors and splices, and judge the dominant contribution. (3)

| Item | Calculation | Loss (dB) |
| --- | --- | --- |
| Fibre | 0.18 × 25 | 4.50 |
| Connectors | 3 × 0.4 | 1.20 |
| Fusion splices | 4 × 0.08 | 0.32 |

| Item | Calculation | Loss (dB) |
| --- | --- | --- |
| Total |  | 6.02 |

The fibre dominates, at 4.5 dB or about 75% of the total. Connectors add about 20% and splices about 5%.

### Received power and margin

Calculate received power in dBm and μW, then determine the available margin above receiver sensitivity and judge whether the link is acceptable. (3)

Pin = 10 log₁₀(2 mW) = +3.01 dBm

Pout = 3.01 − 6.02 = −3.01 dBm = 0.5 mW = 500 μW

Margin = −3.01 − (−20) = 16.99 ≈ 17 dB

The signal arrives about 17 dB above the receiver sensitivity, so the link is acceptable.

### Required margin and the 100 km proposal

Evaluate whether the required 5 dB margin is met and whether the same equipment can support a 100 km route. Estimate the maximum supportable length, ignoring additional splices. (2)

The available 17 dB exceeds the required 5 dB by about 12 dB, so the margin requirement is met for 25 km. For 100 km, fibre loss is 0.18 × 100 = 18 dB. With 1.52 dB of connector and splice loss the total is 19.52 dB, so the received power is −16.51 dBm and the margin is only

3.49 dB. This is below the required 5 dB, so the proposal to reuse the equipment fails. Maximum length, with the 5 dB margin and the same fixed losses:

Lmax = [(3.01 + 20) − 5 − 1.52] / 0.18 = 16.49 / 0.18 ≈ 91.6 km

Recommendation: do not approve 100 km with this equipment. Limit the route to about 91 km, or add an amplifier or a higher-power transmitter.

## Q3. Dispersion Evaluation of a 10 Gb/s Optical Link [BL: K5] (5 marks)

A 40 km single-mode link at 1550 nm carries 10 Gb/s NRZ data. The fiber dispersion parameter is D = 16 ps/(nm·km), and the laser spectral width is 0.10 nm. Use the dispersion-limited criterion B·Δτ ≤ 0.25. A dispersion-compensating fiber (DCF) with D =

−80 ps/(nm·km) is available. The design team claims that compensation is unnecessary because the link distance is moderate.

Summary. The design team's claim fails: spreading is 64 ps against a 100 ps bit period, and the link is limited to about 3.9 Gb/s (or 15.6 km at 10 Gb/s). Compensation or a narrower source is needed.

### Pulse spreading versus bit period

Calculate dispersion-induced pulse spreading over 40 km and compare it with the bit period at 10 Gb/s. Judge the severity of spreading. (2)

Δτ = D L Δλ = 16 × 40 × 0.10 = 64 ps

The bit period at 10 Gb/s is T = 1/B = 100 ps, so the pulse spreads over 64% of a bit. This is severe and causes heavy inter-symbol interference. Check: B·Δτ = 10×10⁹ × 64×10⁻¹² = 0.64, well above the 0.25 limit.

### Assessment of the "no compensation needed" claim

Assess the design team's claim using the dispersion-limited bit rate and maximum link length. Determine whether reducing the source spectral width to 0.02 nm would satisfy the stated criterion. (2)

Bmax = 0.25 / Δτ = 0.25 / 64 ps = 3.9 Gb/s

Lmax = 0.25 / (B D Δλ) = 0.25 / [(10×10⁹)(16×10⁻¹²)(0.10) s/km] ≈ 15.6 km

The link supports only 3.9 Gb/s over 40 km, or 15.6 km at 10 Gb/s. Judgement: the claim is not valid, because a moderate distance is still too long for this laser at 10 Gb/s. With Δλ = 0.02 nm:

Δτ = 16 × 40 × 0.02 = 12.8 ps,	B·Δτ = 0.128 ≤ 0.25

The criterion is satisfied (maximum length about 78 km at 10 Gb/s), so a narrower source solves the problem without a DCF.

### DCF length for full first-order compensation

Calculate the DCF length required for complete first-order compensation and state the condition used. (1)

The condition is that total accumulated dispersion is zero:

D₁L₁ + D_DCF L_DCF = 0 ⇒ L_DCF = (16 × 40) / 80 = 8 km

About 8 km of DCF cancels the dispersion of the 40 km span.