# ECA1803 OPTOELECTRONICS AND OPTICAL COMMUNICATION

CO2 CIA-1 Report: Optical Sources and Nonlinear Effects

Name: Dinesh Kumar	Register No.: 192312169

## Q1. Evaluating an LED Transmitter for a 980 nm Monitoring Link [BL: K5] (10 marks)

A 980 nm LED is driven at 25 mA. Its internal quantum efficiency is 0.65 and its external extraction efficiency is 4%. The active region has a bandgap energy of 1.27 eV. An edge-emitting laser at the same wavelength can couple 3 mW into the fiber and has a spectral width of 2 nm, while the LED has a spectral width of about 35 nm. A proposal recommends using the LED for a 1 Gb/s, 2 km monitoring link to reduce cost.

Summary. The bandgap is suitable for 980 nm, but the LED delivers only 0.82 mW and is too slow and too broad in spectrum for 1 Gb/s over 2 km, so the edge-emitting laser is the right source.

### Bandgap and photon energy

Evaluate whether the stated active-region bandgap is suitable for 980 nm emission by calculating the photon energy and comparing it with the bandgap. (2)

Eph = hc/λ = 1239.8 eV·nm / 980 nm = 1.265 eV λg = 1239.8 / 1.27 = 976 nm

The photon energy (1.265 eV) is only 5 meV below the bandgap (1.27 eV), well inside the thermal spread of about kT = 26 meV at room temperature. The gap corresponds to 976 nm, so it is suitable for 980 nm emission.

### Internal and external optical power

Calculate the LED's internally generated optical power and external output power, then judge the effect of the extraction efficiency on usable power. (3)

Pint = ηint (I/q) hν = 0.65 × 0.025 A × 1.265 V = 20.6 mW

Pext = ηext Pint = 0.04 × 20.6 = 0.82 mW

Only 4% of the generated light escapes. The rest is lost mainly to total internal reflection at the semiconductor surface and is turned into heat. The 0.82 mW is emitted into a wide cone, so the power actually coupled into a fibre is far lower still. Judgement: extraction efficiency is the main limit on usable LED power.

### LED versus edge-emitting laser

Compare the LED and edge-emitting laser in spectral width, coupled power, modulation capability and fiber coupling, relating the differences to spontaneous and stimulated emission. (3)

| Parameter | LED | Edge-emitting laser |
| --- | --- | --- |
| Emission process | Spontaneous, random phase | Stimulated, coherent |
| Spectral width | About 35 nm | About 2 nm (17.5 times narrower) |
| Optical power | 0.82 mW emitted, much less coupled | 3 mW coupled into fibre |
| Modulation capability | Limited by carrier lifetime, low rate | High speed, to multi-GHz |
| Fibre coupling | Wide, diffuse beam, poor | Narrow directional beam, efficient |

Spontaneous emission gives random photons in all directions and a wide spectrum. Stimulated emission in the laser cavity gives directional, coherent light with a narrow line and fast response.

### Assessment of the 1 Gb/s, 2 km proposal

Critically assess the proposal to use the LED for the 1 Gb/s, 2 km link and recommend the more suitable source with evidence. (2)

The proposal is not sound, for three reasons:

  - Speed: an LED's modulation bandwidth is set by carrier lifetime and is typically well below 1 Gb/s.

  - Dispersion: the LED's spectrum is 17.5 times wider than the laser's, so chromatic dispersion spreading is 17.5 times larger for the same fibre and length.

  - Power: the laser couples 3 mW (4.8 dBm), while the LED's coupled power is a small fraction of 0.82 mW (−0.9 dBm at the emitter), which cuts the link budget.

Recommendation: use the edge-emitting laser. The cost saving from the LED does not justify failing the 1 Gb/s requirement.

## Q2. Bias, Modulation Bandwidth and Thermal Stability of a 1550 nm DFB Laser [BL: K5] (10 marks)

A 1550 nm DFB laser has a threshold current of 18 mA at 25 °C and emits 6 mW at 48 mA. Its relaxation-oscillation frequency is 4.2 GHz at 48 mA, and its characteristic temperature is T₀ = 75 K. Assume fᵣ ∝ √(I − Iₜₕ), f₃dB ≈ 1.55 fᵣ, and Iₜₕ(T) = Iₜₕ(25 °C) exp[(T − 25)/T₀]. A 10 Gb/s NRZ link requires approximately 7 GHz bandwidth. The team proposes raising the bias to 60 mA and operating without cooling at 55 °C.

Summary. Raising the bias to 60 mA gives 7.70 GHz at 25 °C, enough for 10 Gb/s, but uncooled operation at 55 °C drops the bandwidth below 7 GHz, and external modulation is preferable for a long 1550 nm link.

### Slope efficiency and differential quantum efficiency

Calculate the slope efficiency and external differential quantum efficiency, and judge the laser's conversion performance. (3)

ηs = P / (I − Ith) = 6 mW / (48 − 18 mA) = 0.20 W/A

ηd = ηs · q/(hν) = 0.20 / 0.80 = 0.25 = 25%

Photon energy at 1550 nm is 0.80 eV. A slope efficiency of 0.2 W/A and 25% differential efficiency is a moderate, normal value for a DFB laser: one in four injected electrons above threshold becomes an output photon.

- Bandwidth at 60 mA (25 °C)

Estimate the relaxation-oscillation frequency and 3-dB bandwidth at 60 mA, then evaluate whether the higher bias provides adequate bandwidth for 10 Gb/s NRZ operation. (3)

fr = 4.2 √[(60 − 18)/(48 − 18)] = 4.2 √1.4 = 4.97 GHz f3dB ≈ 1.55 fr = 1.55 × 4.97 = 7.70 GHz

The requirement is about 7 GHz, so 7.70 GHz is adequate with a small margin of 0.7 GHz (about 10%). At the original 48 mA the bandwidth is only 1.55 × 4.2 = 6.51 GHz, which is not enough, so the higher bias is justified.

- Operation at 55 °C and fixed 48 mA bias

Calculate the threshold current at 55 °C and estimate the output power and modulation bandwidth at the fixed 48 mA bias. Judge the uncooled-operation proposal. (2)

Ith(55 °C) = 18 e^((55−25)/75) = 18 e^0.4 = 26.9 mA

Assuming the slope efficiency stays at 0.2 W/A:

P = 0.2 × (48 − 26.9) = 4.2 mW

fr = 4.2 √(21.1/30) = 3.53 GHz,	f3dB = 1.55 × 3.53 = 5.47 GHz

Output power falls from 6 mW to about 4.2 mW (−29%) and bandwidth falls to 5.47 GHz, below the 7 GHz needed. Judgement: uncooled operation at 48 mA fails. The team's full proposal (60 mA at 55 °C) gives I − Ith = 33.1 mA, so fr = 4.42 GHz and f3dB = 6.84 GHz. That is still about 0.16 GHz short of 7 GHz, and the extra bias current adds heat. Some cooling, or a higher bias, is needed.

### Direct versus external modulation

Evaluate the suitability of direct modulation for a 1550 nm, long-distance link by considering chirp and chromatic dispersion, and recommend whether external modulation is preferable. (2)

Direct modulation changes the injected current, which also changes the carrier density and refractive index, so the optical frequency shifts with the data (chirp). At 1550 nm the standard fibre has a dispersion of about 16 ps/(nm·km), so a chirped, broadened spectrum makes pulses spread quickly and limits the reach of a 10 Gb/s link.

Recommendation: for a long-distance link, use external modulation (an electro-absorption or Mach-Zehnder modulator). The laser runs as a steady source and the modulator imposes the data with very low chirp. Direct modulation is acceptable only for short, low-cost links.

## Q3. Nonlinear Effects in a 40 km WDM Transmission Span [BL: K5] (5 marks)

A 40 km fiber span has attenuation 0.18 dB/km, nonlinear coefficient γ = 1.2 W⁻¹km⁻¹, effective area 90 μm² and Brillouin gain g_B = 4 × 10⁻¹¹ m/W. Each channel is launched at 3 mW. Use L_eff = (1 − e^(−αL))/α, φ_NL = γP L_eff and P_th(SBS) ≈ 21A_eff/(g_B L_eff), converting attenuation from dB/km to km⁻¹. The planner claims that nonlinear effects can be ignored because launch power is only a few milliwatts.

Summary. SPM is negligible (0.070 rad), but the 3 mW launch exceeds the 2.42 mW SBS threshold, so the planner's claim is wrong for a narrow-linewidth carrier.

### Effective length and SPM phase shift

Calculate the effective length and SPM-induced nonlinear phase shift, and explain why the effective length is less than the physical span length. (2)

α = 0.18 / 4.343 = 0.0414 km⁻¹,	αL = 1.658

Leff = (1 − e^−1.658) / 0.0414 = (1 − 0.1905) / 0.0414 = 19.5 km φNL = γ P Leff = 1.2 × 0.003 × 19.5 = 0.070 rad

The effective length is less than 40 km because the signal power decays along the fibre. Nonlinear effects depend on power, so the weaker far end contributes little. Leff is the length of constant-power fibre that would give the same nonlinear effect. A phase shift of 0.07 rad (about 4°) is small for a single channel.

### SBS threshold

Estimate the SBS threshold for a narrow-linewidth continuous-wave carrier, compare it with the 3 mW launch power and recommend a suitable mitigation method. (2)

Pth = 21 Aeff / (gB Leff) = 21 × 90×10⁻¹² m² / [(4×10⁻¹¹ m/W)(19.5×10³ m)] = 2.42 mW The 3 mW launch is about 1.24 times the threshold (0.9 dB above it), so SBS will reflect a significant part of the power back toward the transmitter. The planner's claim that a few milliwatts is safe does not hold for a narrow-linewidth carrier. Recommended mitigation: broaden the carrier linewidth beyond the Brillouin bandwidth, by dithering the laser current or applying phase modulation. Reducing the launch power below about 2.4 mW also works, at the cost of link budget.

### Four-wave mixing products

Two WDM channels are at 193.00 THz and 193.15 THz. Calculate the FWM products 2f₁

− f₂ and 2f₂ − f₁, then judge their potential to cause in-band crosstalk on a 50 GHz grid. (1)

2f₁ − f₂ = 2(193.00) − 193.15 = 192.85 THz

2f₂ − f₁ = 2(193.15) − 193.00 = 193.30 THz

The channel spacing is 150 GHz, so the products fall exactly 150 GHz outside the two channels, which is three slots of the 50 GHz grid. They do not overlap the two existing channels, but they land exactly on grid slots (192.85 and 193.30 THz). If those slots carry channels, the FWM products cause direct in-band crosstalk. Equal channel spacing makes this worst, so unequal spacing or higher local dispersion is advisable.