# ECA1803 OPTOELECTRONICS AND OPTICAL COMMUNICATION

CO3 CIA-2 : Photodetectors, LiDAR and PPG Name: Dinesh Kumar	Register No.: 192312169

## Q1. PIN versus APD for a 1310 nm Optical Receiver:

An InGaAs PIN photodiode with quantum efficiency 78% receives an average optical power of −28 dBm at 1310 nm. The receiver bandwidth is 1.2 GHz, dark current is 8 nA, load resistance is 750 Ω and temperature is 300 K (k = 1.38 × 10⁻²³ J/K). An APD option has gain M = 12 and excess noise factor F = M^0.4. The designer proposes using an APD at M = 12 and claims that increasing the gain to M = 60 must further improve sensitivity.

Summary. The PIN receiver reaches an SNR of 18.0 dB and is thermal-noise limited. The APD at M = 12 raises this to 30.4 dB (a 12.4 dB gain), but at M = 60 the SNR falls to 28.2 dB, so the claim that more gain always helps is false.

## PIN responsivity and photocurrent

Calculate the PIN responsivity at 1310 nm and the primary photocurrent at −28 dBm; compare the responsivity with the ideal quantum-efficiency value and judge its suitability. (3)

R = ηqλ/(hc) = 0.78 × 1.31 μm / 1.24 μm·V = 0.824 A/W P = 10^(−28/10) mW = 1.585 μW

Iₚ = R·P = 0.824 × 1.585 μW = 1.31 μA

The ideal responsivity (η = 1) is 1.31/1.24 = 1.057 A/W, so the actual value is 78% of the ideal. A responsivity of 0.82 A/W is typical of a good InGaAs PIN at 1310 nm, so the device is suitable.

## Noise currents and PIN SNR

Evaluate the shot-noise and thermal-noise mean-square currents and the PIN receiver SNR in dB, then identify the dominant noise source. (3)

⟨iₛ²⟩ = 2q(Iₚ + I_d)B = 2(1.602×10⁻¹⁹)(1.31 μA + 8 nA)(1.2×10⁹) = 5.05×10⁻¹⁶ A²

⟨i_T²⟩ = 4kTB/R_L = 4(1.38×10⁻²³)(300)(1.2×10⁹)/750 = 2.65×10⁻¹⁴ A²

SNR = Iₚ² / (⟨iₛ²⟩ + ⟨i_T²⟩) = 1.706×10⁻¹² / 2.70×10⁻¹⁴ = 63.2 = 18.0 dB

Thermal noise is about 52 times larger than shot noise (98% of the total), so the PIN receiver is thermal-noise limited.

- APD at M = 12

Calculate the APD excess noise factor and SNR at M = 12, and quantify the improvement over the PIN receiver. (2)

F = M^0.4 = 12^0.4 = 2.70

SNR = M²Iₚ² / (M²F⟨iₛ²⟩ + ⟨i_T²⟩) = 2.457×10⁻¹⁰ / 2.231×10⁻¹³ = 1101 = 30.4 dB

The improvement over the PIN is 1101/63.2 = 17.4 times, or 12.4 dB. (Dark current is treated as multiplied by the gain, a standard simplification.) The gain lifts the signal above the fixed thermal noise.

- APD at M = 60 and the gain claim

Evaluate the SNR at M = 60 and critique the claim that higher APD gain always improves sensitivity. (2)

F = 60^0.4 = 5.14

SNR = 3600 × 1.706×10⁻¹² / (3600 × 5.14 × 5.05×10⁻¹⁶ + 2.65×10⁻¹⁴) = 6.14×10⁻⁹ /

9.38×10⁻¹² = 655 = 28.2 dB

The SNR falls by 2.3 dB compared with M = 12. At this gain the multiplied shot noise (with its excess noise factor) exceeds the thermal noise, so extra gain amplifies noise faster than signal. Setting d(SNR)/dM = 0 gives M^(2+x) = 2⟨i_T²⟩/(x⟨iₛ²⟩) with x = 0.4, which yields an optimum gain M_opt ≈ 10 (SNR about 30.5 dB). Judgement: the designer's claim is wrong; sensitivity improves only up to an optimum gain, and M = 12 is already close to it.

## Q2. Bandwidth Assessment of a Photodiode for 20 Gb/s Data:

A PIN photodiode has a depletion width of 0.8 μm, carrier saturation velocity 8 × 10⁴ m/s, junction capacitance 0.18 pF and load resistance 50 Ω. Use f_tr ≈ 0.44/t_tr, f_RC = 1/(2πRC), and 1/f² = 1/f_tr² + 1/f_RC². A supplier claims that the detector can support both 10 Gb/s and 20 Gb/s NRZ links, for which the required bandwidth is approximately

0.7 times the bit rate.

Summary. The detector is RC-limited at 17.7 GHz and has an overall bandwidth of 16.4 GHz. That meets 7 GHz (10 Gb/s) comfortably and 14 GHz (20 Gb/s) only narrowly, so a lower junction capacitance is advisable for 20 Gb/s.

## Transit-time limit

Calculate the carrier transit time and transit-time-limited bandwidth, and judge whether transit time restricts the design. (3)

t_tr = W / v_s = 0.8×10⁻⁶ / 8×10⁴ = 10 ps f_tr = 0.44 / t_tr = 0.44 / 10 ps = 44 GHz

A transit-limited bandwidth of 44 GHz is well above the 14 GHz needed, so transit time does not restrict the design.

## RC limit

Calculate the RC time constant and RC-limited bandwidth, and identify the dominant bandwidth limitation. (3)

RC = 50 Ω × 0.18 pF = 9 ps

f_RC = 1 / (2π × 9 ps) = 17.7 GHz

The RC limit (17.7 GHz) is much lower than the transit limit (44 GHz), so the junction capacitance and load resistance are the dominant limitation.

## Overall bandwidth and the supplier's claim

Determine the overall bandwidth and assess the supplier's claim for both 10 Gb/s and 20 Gb/s operation. (2)

1/f² = 1/(44)² + 1/(17.7)² = 5.17×10⁻⁴ + 3.20×10⁻³ = 3.71×10⁻³ GHz⁻² f = 16.4 GHz

| Bit rate | Required bandwidth (0.7 B) | Available (16.4 GHz) | Margin |
| --- | --- | --- | --- |
| 10 Gb/s | 7 GHz | 16.4 GHz | 9.4 GHz, comfortable |
| 20 Gb/s | 14 GHz | 16.4 GHz | 2.4 GHz (17%), narrow |

The claim holds for 10 Gb/s. For 20 Gb/s it is only marginally true: a 17% margin leaves little room for package parasitics and tolerances, so the claim is weak.

## Redesign for 20 Gb/s

Recommend a practical redesign for 20 Gb/s operation by evaluating the trade-off among junction capacitance, depletion width, optical absorption and responsivity. (2)

Since the RC term dominates, the best step is to cut the junction capacitance by reducing the active area. Lowering C to about 0.10 pF gives RC = 5 ps and f_RC = 31.8 GHz, so the overall bandwidth becomes about 25.8 GHz, a comfortable margin over 14 GHz.

  - Depletion width: thinning W shortens the transit time but raises C (C ∝ A/W), so it works against the RC limit.

  - Absorption and responsivity: quantum efficiency follows η = 1 − e^(−αW), so a thinner W absorbs less light and lowers responsivity.

  - Smaller area: a small area lowers C but makes optical alignment and coupling harder, so a lens or an edge-coupled waveguide photodiode may be needed to keep responsivity.

Recommendation: keep W near 0.8 μm to preserve responsivity and reduce the area to bring C to about 0.1 pF.

## Q3. LiDAR Range Resolution and Wearable PPG:

An automotive LiDAR receives a return pulse 520 ns after transmission of a 4 ns laser pulse. A team claims this is sufficient to distinguish two road users separated by 0.5 m. A wearable PPG sensor uses a silicon photodiode with responsivity 0.50 A/W; its received optical signal has a DC component of 10 μW and a pulsatile AC component of 30 nW. The LiDAR team proposes a PIN photodiode for detecting weak returns in bright sunlight.

Summary. The target is at 78 m, but the 4 ns pulse limits resolution to 0.6 m, so the 0.5 m claim fails. The PPG perfusion index is 0.3%, and a PIN detector is a poor choice for weak sunlit echoes.

## Range and resolution

Calculate the target range and pulse-width-limited range resolution, then judge whether the 0.5 m separation claim is justified. (2)

R = ct/2 = (3×10⁸ × 520×10⁻⁹) / 2 = 78 m ΔR = cτ/2 = (3×10⁸ × 4×10⁻⁹) / 2 = 0.6 m

The 520 ns delay only gives the range. The resolution depends on the pulse width, and 0.6 m is larger than 0.5 m, so the two road users would merge into one echo. The claim is not justified; a pulse of at most 2 × 0.5 / (3×10⁸) = 3.3 ns would be needed.

## PPG photocurrents and perfusion index

Calculate the DC and AC photocurrents and the perfusion index, and explain the physiological origin of the AC component. (2)

I_DC = 0.50 × 10 μW = 5 μA

I_AC = 0.50 × 30 nW = 15 nA

PI = I_AC / I_DC × 100 = 15 nA / 5 μA × 100 = 0.3 %

The AC component comes from the pulsatile arterial blood volume. With each heartbeat the arteries fill with blood, which absorbs more light (haemoglobin absorbs strongly), so the detected light dips and recovers at the heart rate. The DC component comes from static tissue, bone, venous blood and the non-pulsatile blood.

## PIN detector for LiDAR in sunlight

Critique the use of a PIN detector for weak LiDAR echoes in sunlight and recommend one alternative detector and one background-rejection method. (1)

A PIN diode has no internal gain, so a weak echo gives a photocurrent below the amplifier's thermal noise, while bright sunlight adds shot noise and a large DC background that can saturate the front end. The SNR is poor. The recommended alternative is an avalanche photodiode (APD), whose gain lifts the echo above the amplifier noise. For background rejection, use a narrow-band optical filter centred on the laser wavelength to block most of the sunlight.